USER  MANUAL

Digital Read Out with RS485 Output (MOD-BUS)

Front View				    Back View

FEATURES:

It counts the incoming pulses, multiplies counts with specified scale factor.

Device maintains its parameter and state of Output in case of power failure.

SPECIFICATIONS:

DISPLAY  :  Six Digit Seven Segment Display, Red Colour

SENSOR:   Rotary Encoder with Two Channels

SUPPLY:   24V DC + 15%

DIMENSIONS: (W x H x D) (96 x 96 x 110)mm

PANEL CUTOUT:   (92 x 92)mm

MEMORY BACKUP :   Non- Volatile

Abbreviations:

ACK 	:	Acknowledge .To Save Current Configuration.

SF	            :	Scale Factor (As per formula given below)

PS		: 	Pre-set Value (000000 – 999999), set by pressing pre-set/recall key.

DP		:	Dot Point

Reset  	: 	Count can be reset to zero and all outputs switched off.

S.F. 		:           Scale factor

LT 		:	NOT USED

Protocol Setting for Serial Output :

Bit Per Second(BPS)       :   9600

Data Bits                          :      8

Parity                               :   None

Stop Bit                            :      1

Flow Control                    :   None

PAGE 1OF 3

Working:-

MODE/ACK:  This key is used to save the current configuration for corresponding parameters. Press         		 Key for 2 Sec it will move to Programming Mode. To move to setting Indication press 		              mode key till it selects the required Parameter of setting Indication.

Esc.             : This key is used to clear the previous reading.

EDIT :          To edit the parameters press the mode key to select the required parameter and then press 	              Edit  Key.

INC/RESET:   In normal running mode this Key is used to reset the current counts to zero (counts=0)

Increments the selected digit value from 0-9 when in configuration mode.

DOT POINT: This Key  is used for dot point setting. To change the position of Dot Point, press Dot  		              point Key, the corresponding Led will glow. Then select the position of the Dot point as

required

Example      :  If you want to have decimal point in second place then dot point should be 0.00

SHIFT/PRESET RECAL:    In normal display mode this key will display (000000)Press Key, the 	  	                              corresponding Led will glow and the previous stored Value will display. 				     The value will  move from the value saved in  PS

Scale Factor	:    The formula to calculate the scale factor

Scale factor =       Reading required per revolution

4 x PPR of encoder

Example:

Suppose we have a 500 PPR encoder and we want the reading per revolution to be 500

Scale factor =      500     =   000.250     OR we can feed  0.25000 for better calculations.

4 x 500

Similarly with a 500 PPR encoder to DISPLAY a reading of 20/rev  : scale factor = 0.01

LIMIT    : When this key is pressed for more than 3 second user is able to enter the Limit mode. 		     The corresponding Leds will glow. Now enter the value for Limit1 and press ACK 		     Key to save the Limit1 value. After Pressing Ack. Key user will be in Limit 2 			     configuration mode, Enter Limit2 Value, again press ACK Key to save Limit2 Value 		     and to enter running mode.

Example : Limit 1(LT1) = 100   Limit 2 (LT2) = 200, Timing = 00

Set LTI=100 When the count read 100 then Limit1 operate and the Led LT1 will glow.

Set LT2=200 When the count read 200 then Limit 2 operate and the LT2 will glow and both the Relays operate & Leds will glow till the RESET . This mode is Latching Mode

Example : Limit 1(LT1) - 100   Limit 2 (LT2) -200 Timing = 1Sec to 10 Sec

Set  LTI-100 and Ack and Set  LT2-200 and Ack & time 1 Second & Ack. When the count will read 100 the Relay 1 operate & Led LT1 will glow and when the count reads 200 Relay 2 operate & LT2 will glow and both the Relay operate &  Leds will glow till  1-sec. This is Monoshot mode.

PAGE 2 OF 3

Protocol Setting for Modbus DRO  :

Holding Registers

40001 : read/write the value lower 16 bit

40002 : read/write the value upper 16bit

40003 : read the address of node

40004 : Change the address of node

baud rate : 9600

word length : 8

parity : none

stop bit : 1

Address : 127 (Master address)

Setup :

1. Connect the DRO with computer with RS-485 using USB to RS-485 converter.

2. Open Mode Scan

3. Select Device ID 127, Length 4, holding register.

4. Click on connections 	connect. New window will be open.

5. Select comport, baud rate 9600, word length 4, parity none and stop bit as 1.

6. you will get the DRO value in address 40001 and 40002 in hexadecimal.

7. To reset the counts write 0 in address 40001 and 40002

8. Device ID shown in the address 40003 of that particular node.

9. Device ID  can be changed by click on address 40004.

10. Convert the hexadecimal value to decimal that is displayed on the DRO.

11. Range of DRO is 0 to 999999(F423F in Hexadecimal).

Note:- To change the address and 1st time communication. Connect 1 pc at a time.

PAGE 3 OF 3